Smithson’s Food Trial Report

| Date | 21/07/2022 |
| --- | --- |
| Line | Line 1 |
| Product | Pork Steaks 400g |
| Trial Objective | To trial a new slicer machine on Line 1 |
| Trial Settings | Pack Weight: 400g Slice Weight: 100g Slices per pack: 4 Meat Slicing temperature: 5oC Line speed: 5 |
| Trial Outcome | Product sliced well Initial settings had to be changed by engineers as the cut meat was very messy. Line speed was also increased to 8 as the slicer is quicker than current line setting The product ran for the whole shift with no issues |
| Actions | Adjust line speed in product settings Get engineers to set program to match settings for neat cut |

| Date | 22/07/2022 |
| --- | --- |
| Line | Line 2 |
| Product | Pork Medallions 600g |
| Trial Objective | To trial a new slicer machine on Line 2 |
| Trial Settings | Pack Weight: 600g Slice Weight: 100g Slices per pack: 6 Meat Slicing temperature: 5oC Line speed: 6 |
| Trial Outcome | Product sliced well Line speed was increased to 8 as the slicer is quicker than current line setting. The initial settings damaged the blue belt of the line, engineers were called to fix the belt but some damage caused blue shards to be broken off the belt. Engineer had to stop line and place hand through guard to fix the blade. Full line shut down was not completed by the engineer before placing hand past guard. Good quality product, and throughput was increased |
| Actions | Adjust line speed in product settings Get engineers to set program to match settings to prevent damage to line. Training of staff required for new settings |

| Date | 23/07/2022 |
| --- | --- |
| Line | Line 3 |
| Product | Diced Chicken Breast 500g |
| Trial Objective | To trial a new slicer machine on Line 3 |
| Trial Settings | Pack Weight: 500g Slice Weight: 25-45g Slices per pack: N/A as diced Meat Slicing temperature: 5oC Line speed: 6 |
| Trial Outcome | Product was very messy on slice and overweight. Product temperature was reduced to 1oC giving a much neater cut on the product. Quality and process checks confirmed overweight product, settings adjusted to bring average pack weight down. Line speed remained the same, but packs per minute efficiency increased as smaller gap between portions on belt. |
| Actions | Get engineers to set program to match settings to prevent product giveaway and lower temperature for better slicing. Training of line staff required for new settings |

| Date | 24/07/2022 |
| --- | --- |
| Line | Line 4 |
| Product | Beef Steaks 240g |
| Trial Objective | To trial a new slicer machine on Line 4 |
| Trial Settings | Pack Weight: 240g Slice Weight: 120g Slices per pack: 2 Meat Slicing temperature: 5oC Line speed: 6 |
| Trial Outcome | Product was initially a messy slice. Temperature was reduced to 1oC to give a much neater slice. Line speed was increased to 8 as the slicer is quicker than current line setting. Good quality product, and throughput was increased |
| Actions | Adjust line speed in product settings Get engineers to set program for lower temperature for better slicing. Training of staff required for new settings |