Assessment guide

Working in the health and science sector

Health and science: science

Science area of study assessment

SAMPLE

# Working in the health and science sector

Health and science: science

# Science area of study assessment

  - Sample

# To be given to learners on or after XX XXXXXXXX XXXX.

# Learner instructions

read all provided information carefully before you start the work

you must clearly identify and label all of the work you produce during the supervised time

# Learner information

the maximum time for this assessment is 3 hours 15 minutes

all of the work you submit must be your own

# Please complete the details below clearly and in BLOCK CAPITALS.

| # Learner name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Centre name |  |  |  |  |  |
| # Learner number |  |  | # Centre number |  |  |  |

# Planning scenario

You work in a laboratory which measures the nutritional value of beverages. You have been asked to design a laboratory protocol which will measure mass of vitamin C levels within different soft drinks. Vitamin C is a key component of a balanced diet however some people with chronic illnesses need to monitor their vitamin intake carefully, so this is vital information.

Dissolved vitamin C reacts with a chemical called DCPIP. DCPIP is blue when dissolved in water however when it is reduced by vitamin C it becomes a colourless compound. Vitamin C can be used in a titration reaction with DCPIP. The end point can be seen when DCPIP turns from blue to colourless.

Vitamin C and DCPIP react in a one-to-one ratio during the titration. The volume used to reduce DCPIP can vitamin C can be used to work out the mass of vitamin C the soft drinks in mg/cm3.

How is mass of vitamin C calculated from the titration?

A 1% solution vitamin C will be the equivalent of 10 mg cm-3 of vitamin C. By doing a titration using can work out the mass of vitamin C using the following equation.

Mass of vitamin C = 10mg × volume of vitamin C used during the titration against DCPIP to turn colourless.

# Task 1: planning

| Maximum time: | 1 hour |
| --- | --- |
| Resource list | practical plan template (appendix 1) risk assessment template (appendix 2) |
| You are required to: For your plan you will plan a titration experiment using 1% solution of DCPIP and a 1% solution of the fruit juice. These solutions will be prepared for you. You will need to complete a risk assessment (template provided in appendix 1) on health and safety identifying all the hazards in the lab such as from electrical sockets and the solutions used. You will also consider the correct personal protective equipment (PPE)/hazard symbols on the risk assessment template which will be given in the instruction sheets. There are also datasheets on the chemicals used in the practical and hazards symbols. In your plan you need to include suitable equipment which is accurate and precise and how the levels of the liquid could be measured. You should also consider what would be part of the standard operating procedure such as good housekeeping skills, suitable PPE and how any accidents should be handled correctly. |  |
| Evidence | planning documents risk assessment |

# Practical scenario

You are a technician in a company that wishes to alter the structure of its electrical circuits due to new safety rules which mean that workstations must be further apart. One issue that the company faces is that if it extends workstations this may mean that the wires in some electrical circuits may have to be further apart.

By testing resistance, you are testing the condition of a circuit or component (the higher resistance, the lower the current flow). Resistors will restrict the flow of electricity and protect components. A measure of resistance of an unknown conductor can be important in determining whether that material is suitable to use and if the length will impact on the circuit’s performance.

### Circuit board diagram

www.bbc.co.uk bitesize GCSE Physics

# Task 2: practical

| Maximum time: | 1 hour 30 minutes |
| --- | --- |
| Resource list: | standard operating procedure (appendix 3) table and graph guidance (appendix 4) |
| Equipment list: | voltmeter circuit board wires with crocodile clips ammeter plastic ruler minimum length 1000mm 1.5 V cell |
| ### You are required to: Follow the practical below following health and safety regulations and records your results in a table following standard operating procedure. You need record the length of the wire accurately so you can measure the potential difference (voltage) and current (amperes). set up the parallel circuit using the circuit board diagram given in the scenario connect first crocodile clips to the resistance wire at 0 cm connect second clip at 100 mm make sure the switch is shut and the wires are not hold record the reading on the ammeter and on the voltmeter in a suitable table with a title, suitable labelling of headings with suitable units repeat the previous steps increasing the length of the wire by 100 mm each time down to a minimum length of 1000 mm use the results to calculate the resistance of each length of wire, where R is resistance, V is potential difference, and I is current |  |
| Evidence | table of results |

# Data analysis scenario

Blood samples are taken in hospitals and vets to measure the concentration of red blood cells, and sent to specialist laboratories for analysis. Red blood cells carry oxygen and a complete blood count allows a technician to see if people or animals have certain conditions such as anaemia. Complete blood cell count (CBC) looks at the number of white blood cells, the number of platelets and the number of red blood cells in a small volume of blood taken from a patient. For safety and accuracy most labels will use machinery called automated cell counters. The machine requires careful handling and cleaning and must be checked at the end of each batch of counting so that there is no contamination or error.

You work within the quality assurance team which takes randomised samples to make sure complete blood counts are accurate. Recently the readings on the automated cell counter on laboratory information management system (LIMS) have alerted your team leader.

You have been asked to identify which results may be cause concern and what could cause these results. You have been asked to consider why these results may pose issues and how these results could be avoided. As part of your job, you need to make sure that CBC is completed, and you need to suggest a suitable alternative practical and the steps needed to be taken out on the blood so it can be used in this alternative method.

Table to show red blood cell count of different samples taken from the machine

| Sample of blood (human) | Number of red blood cells per mm3 | Suitable range |
| --- | --- | --- |
| A | 5 000 000 | 4 million to 6 million/mm3 |
| B | 4 560 000 | 4 million to 6 million/mm3 |
| C | 6 300 000 | 4 million to 6 million/mm3 |
| D | 5 900 000 | 4 million to 6 million/mm3 |
| E | 3 980 000 | 4 million to 6 million/mm3 |
| F | 100 000 | 4 million to 6 million/mm3 |
| G | 6 700 000 | 4 million to 6 million/mm3 |
| H | 3 500 000 | 4 million to 6 million/mm3 |

# Task 3: data analysis

| Maximum time: | 45 minutes |
| --- | --- |
| You are required to: Review the data answer and answer the following questions: state which result from the table is incorrect explain what could have caused these results to happen explain would you do to amend the incorrect result – suggest what could have happened to the machine describe the alternative method to counting red blood cells this way explain the steps need to be taken in order to allow this blood to be counted describe how the blood is prepared for counting – state the name of the process and describe any calculations which are needed for this step |  |
| Evidence | Documents showing responses to assessment questions |

# Appendix 1: practical plan

Title

Aim of practical

What are the housekeeping rules that should be followed when carrying out the practical?

How do people protect themselves when working with chemicals and within the laboratory setting?

Equipment list to be used in practical – think about accuracy

Step by step instruction for practical

Table of results: please draw a table where the results for the titration could be stored and the units for the table.

# Appendix 2: risk assessment for titration practical to measure mass of vitamin C with DCPIP

| Date: |  |
| --- | --- |
| Experiment: |  |
| Author: |  |

| Hazard (chemical, action, lab) Please include the correct symbol | Risk of harm | Steps to prevent risk of hazards |
| --- | --- | --- |

Questions to consider with risk assessment

What are the general hazards in the laboratory?

What are the main laws which regulate safety in the laboratory?

Where do we find information about the risks of the chemicals that we are using during this experiment?

# Appendix 3: standard operating procedure

equipment should be PAT tested and in working order with no loose wires or frayed cables

all wires and equipment should be shiny and have no rust

all trip hazards should be removed from desks

all spills should be wiped up to remove risk of trips

suitable PPE should be worn

wires should be left to cool between each experiment

all circuits should be checked to make sure power sources is not near water or broken

wire used should not be broken

no trip hazards should be removed.

learners should wear lab coats and tie back long hair

# Appendix 4: table and graph guidance

Using your results, please consider the following questions when formatting your data from the experiment.

How should you draw a table?

What different units are you measuring?

What units have you measured?

What is the independent variable and what is the dependent variable?

How do you measure resistance from this experiment?

What would you plot on the graph, which value would go on the X or Y axis?

# Document information

All the material in this publication is © NCFE

Owner: Head of Assessment Design